CPE 166 Midterm 2 Review

Picture of Logic Gates

VHDL Syntax

Assign: <=

Compare: =

Numbers: need to be in ‘’ or “”

Logical Operators

And: c <= a and b;

Or:  c <= a or b;

Nand: c < a nand b;

Nor: c < a nor b;

Xor: c <= a xor b;

Xnor: c <= a xnor b;

Not: c <= not c;

Relational Operators

=      Equal

/=     Not Equal

<      Less Than

<=     Less Than or Equal To

>      Greater Than

>=     Greater Than or Equal To

Combinational Logic  Designs in VHDL

Half adder

Adder without carry in, sum is 1 when there is a single 1, 0 if two 0’s or 2 1’s. When there are two ones, carry out. Therefore, xor & and gate for HA

library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; entity half_adder is port ( A, B: in std_logic; -- o_sum : out std_logic; o_carry : out std_logic ); end half_adder; architecture rtl of half_adder is begin o_sum <= A xor B; o_carry <= A and B; end rtl;

Full Bit Adder

Adds carry in which changes the combinational logic
Two Half adders create a full adder

library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; entity full_adder is port ( A,B : in std_logic; i_carry : in std_logic; -- o_sum : out std_logic; o_carry : out std_logic ); end full_adder; architecture rtl of full_adder is signal w_WIRE_1 : std_logic; signal w_WIRE_2 : std_logic; signal w_WIRE_3 : std_logic; begin w_WIRE_1 <= A xor B; w_WIRE_2 <= w_WIRE_1 and i_carry; w_WIRE_3 <= A and B; o_sum <= w_WIRE_1 xor i_carry; o_carry <= w_WIRE_2 or w_WIRE_3; end rtl;

Ripple Carry Adder comprises of Full Adders that go into each other

library ieee; use ieee.std_logic_1164.all; entity ripple_carry_adder is generic ( g_WIDTH : natural := 2 ); port ( i_add_term1 : in std_logic_vector(g_WIDTH-1 downto 0); i_add_term2 : in std_logic_vector(g_WIDTH-1 downto 0); -- o_result : out std_logic_vector(g_WIDTH downto 0) ); end ripple_carry_adder; architecture rtl of ripple_carry_adder is component full_adder is port ( A,B : in std_logic; i_carry : in std_logic; o_sum, o_carry : out std_logic; end component full_adder; signal w_CARRY : std_logic_vector(g_WIDTH downto 0); signal w_SUM : std_logic_vector(g_WIDTH-1 downto 0); begin w_CARRY(0) <= '0'; -- no carry input on first full adder SET_WIDTH : for ii in 0 to g_WIDTH-1 generate i_FULL_ADDER_INST : full_adder port map ( A => i_add_term1(ii), B => i_add_term2(ii), i_carry => w_CARRY(ii), o_sum => w_SUM(ii), o_carry => w_CARRY(ii+1) ); end generate SET_WIDTH; o_result <= w_CARRY(g_WIDTH) & w_SUM; -- VHDL Concatenation end rtl;

subtractor

Subtractor is just an adder but with B xor’d

1-b.    Multiplexer

Device that selects different inputs and has a single output. Code:

library ieee; use ieee.std_logic_1164.all; entity mux is port ( a, b, c, d : in std_logic; s : in std_logic_vector (1 downto 0); o : out std_logic ); end mux; architecture archi of mux is begin process (a, b, c, d, s) begin case s is when "00" => o <= a; when "01" => o <= b; when "10" => o <= c; when others => o <= d; end case; end process; end archi;

Decoder

Decoder takes n input and expands to 2^n output. 2to4: 00= 0000, 01 = 0010, 10 = 0100, 11 - 1000 Code:

library ieee; use ieee.std_logic_1164.all; entity dec is port ( sel: in std_logic_vector (2 downto 0); res: out std_logic_vector (7 downto 0) ); end dec; architecture archi of dec is begin with sel select res <= "00000001" when "000", "00000010" when "001", "00000100" when "010", "00001000" when "011", "00010000" when "100", "00100000" when "101", "01000000" when "110" "10000000" when others; end archi;

Comparators

VHDL: (=, /=,<, <=, >, >=)

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; entity compar is port( A, B : in std_logic_vector(7 downto 0); CMP : out std_logic ); end compar; architecture archi of compar is begin CMP <= '1' when A >= B else '0'; end archi;

Sequential circuits using VHDL

D Flip-flops

Flip-flop with Positive-Edge Clock and Synchronous Set

library ieee; use ieee.std_logic_1164.all; entity flop is port( C, CE, PRE : in std_logic; D: in std_logic_vector(3 downto 0); Q : out std_logic_vector (3 downto 0) ); end flop; architecture archi of flop is begin process (C, PRE) begin if (PRE='1') then Q <= "1111"; elsif (C'event and C='1')then if (CE='1') then Q <= D; end if; end if; end process; end archi;

Shift-registers

8-bit Shift-Left Register with Positive-Edge Clock, Serial In, and Serial Out

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity shift is port( C, SI : in std_logic; SO : out std_logic ); end shift; architecture archi of shift is signal tmp: std_logic_vector(7 downto 0); begin process (C) begin if (C'event and C='1') then for i in 0 to 6 loop tmp(i+1) <= tmp(i); end loop; tmp(0) <= SI; end if; end process; SO <= tmp(7); end archi;

Counters .4-bit up/down counter with an asynchronous clear

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity counter is port( C, CLR, UP_DOWN : in std_logic; Q: out std_logic_vector(3 downto 0) ); end counter; architecture archi of counter is signal tmp: std_logic_vector(3 downto 0); begin process (C, CLR) begin if (CLR='1') then tmp <= "0000"; elsif (C'event and C='1') then if (UP_DOWN='1') then tmp <= tmp + 1; else tmp <= tmp - 1; end if; end if; end process; Q <= tmp; end archi;

Clock-dividers

Create a new clock that is a division of the original clock. Code below divides by 100, if you want to change the amount divide, change counter. Counter is number you want to divide -1, elsif counter is half of the number you want to divide by -1

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; entity module_new_clock is port( clk: in std_logic; reset: in std_logic; clkout: out std_logic ); end module_new_clock; architecture beh of module_new_clock is signal cnt: std_logic; signal counter: std_logic_vector(7 downto 0); begin clkout <= cnt; process(clk,reset) begin if (reset='1') then counter<=(others=>'0'); cnt<='0'; elsif( rising_edge(clk) ) then if (counter = 99 ) then counter <= ( others => '0' ); cnt <= '1'; elsif (counter < 499) then counter <= counter + 1; cnt <= '1'; else counter <= counter + 1; cnt <= '0'; end if; end if; end process; end beh;

Be able to develop hierarchical design using VHDL

Finite State Machine

Be able to Draw ASM chart, as well as Finite state machine / ASM Chart
simulation waveforms: Be able to draw current state, next state and also moore outputs, mealy outputs waveforms.

ASM Diagram Block

ASM Rules:

•The entrance paths to an ASM block lead to only one state box

•Of 'N' possible exit paths, for each possible valid input combination, only one exit path can be followed, that is there is only one valid next state.

Waveforms:

8. Be able to draw synthesized circuits with given VHDL codes.

Even parity and odd parity

Even parity is 1 when  there is an odd number of 1s

Odd Parity is 1 when when there is an even number of 1s

Even Parity and Odd Parity Examples

ab   parity  (f_even)

00_0, 01_1, 10_1, 11_0

f_even <= a xor b;

ab   parity  (f_odd)

00_1, 01_0, 10_0,11_1

f_odd <= not ( a xor b);

(7,4) Hamming codes

D is the input for hamming, P is parity bit

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; entity module_haming is port(s0,s1,s2: in std_logic; d: in std_logic_vector(4 downto 1); h: out std_logic_vector(7 downto 1) ); signal p: std_logic_vector(7 downto 1); end module_haming; architecture beh of module_haming is begin process(d,s0,s1,s2) begin if (s0='1') then h<=("0000000"); else p(1)<=d(4) xor d(2) xor d(1); p(2)<=d(4) xor d(3) xor d(1); p(3)<=d(1); p(4)<=d(4) xor d(3) xor d(2); p(5)<=d(2); p(6)<=d(3); p(7)<=d(4); end if; if (s2='1') then h<=p; elsif(s1='1') then h<=('0'&'0'&'0'&d); end if; end process; end beh;

Can check for a single bit error by checking parity

LFSR

Linear Feedback Shift Register generates inputs, can be represented as 1+ x+x^4

LIBRARY IEEE; USE IEEE.STD_LOGIC_1164.ALL; ENTITY lfsr IS PORT ( reset, clk: IN STD_LOGIC; Q : OUT STD_LOGIC_VECTOR(4 downto 1) ); END lfsr; ARCHITECTURE beh OF lfsr IS signal W: std_logic_vector(4 downto 1); BEGIN process( clk, reset ) begin if (reset='1') then W <= ( 1=>'1', others => '0' ); elsif (rising_edge (clk)) then W <= W(3 downto 2) & ( W(1) xor W(4) ) & W(4); end if; end process; Q <= W; END beh; ------------------------------------ LIBRARY IEEE; USE IEEE.STD_LOGIC_1164.ALL; -- if using more libraries add more here ENTITY lfsr_tb is End lfsr_tb; ARCHITECTURE beh OF lfsr_tb IS -- all of the stuff needed to test goes into a component Component lfsr PORT ( reset, clk: IN STD_LOGIC; Q : OUT STD_LOGIC_VECTOR(4 downto 1) ); END Component; signal reset, clk: std_logic; signal Q: std_logic_vector(4 downto 1); BEGIN -- Instantiate the Unit Under Test (UUT) uut: lfsr port map( reset=>reset, clk=>clk, Q=>Q ); -- Clock process definitions Process Begin Clk <= '0'; Wait for 10 ns; Clk <= '1'; Wait for 10 ns; End process; -- Stimulus process Process Begin reset <='1'; Wait for 6 ns; reset <='0'; Wait for 40 ns; Wait; End process; END beh;

Be able to analyze LFSR outputs clock cycle by clock cycle.

CRC Computation

message represented by some polynomial G(x), and a generating polynomial P(x)

1 - Multiply the message G(x) by x^3, where 3 is the number of bits in the CRC.Add 3 three zeros to the binary G(x).

2 - Divide the product x3 [G(x)] by the generating polynomial P(x). (when dividing, xor the two numbers)

We wish to find "the remainder, modulo P(x)"CRC Error

3 - Disregard the quotient and add the remainder C(x)to the product x^3 [G(x)] to yield the codemessage polynomial F(x), which is represented as:

F(x) = x3 [G(x)] + C(x)

ex) G(x) = 110010, P(x) = 1001(x^3 + x^2 + 1)

G(x)x3 = 110010000

Cx = 100

CRC Detection

Upon reception, the entire received F(x) = "message + crc" can be checked simply by dividing F(x)/P(x) using the same generating polynomial. If the remainder after division equals zero, then no error was found.

Metastability

Metastability is when a system persists for an unbounded time in an unstable equilibrium or metastable state. When a digital signal is not 1 or 0 for an example, circuit can act in unpredictable ways.

Tmet = settling time

Tco- clock to output delay(minimum output

transition time)

Th- minimum hold time

Tsu = minimum setup time

MTBF provides an estimate of the mean time between the probable occurrence of two successive metastable events.

You can avoid metastability’s negative effects by using the output of the synchronizing flip flop rather than the asynchronous signal or by adding the Tmet calculated for a specific MTBF to the worst-case timing delay calculations, giving the output of the flipflops time to settle

Hazard

- A glitch is an unwanted pulse at the output  of a combinational logic network – a momentary change in an output that should not have changed

-A circuit with the potential for a glitch is said to have a hazard.In other words a hazard is something intrinsic about a circuit; a circuit with hazard may or may not have a glitch depending on input patterns and the electric characteristics of the circuit

(static 1 hazard, static 0 hazard, dynamic hazard)